FACILITATOR GUIDE · INTERNAL USE ONLY OPERATION GAS VALVE 3-Day Robotics Challenge · Facilitator Overview 7 Teams · 8 Hrs/Day · 3 Days · 2 Robot Builds · 1 Mission

MISSION PREMISE A gas leak has been detected in an inaccessible room. Kelsey, a temp, is on-site but cannot safely enter. Participants, operating from AU as Mission Control, must build a remote-controlled robot AND relay instructions for Kelsey to build an identical one. On Day 3 they remotely pilot Kelsey's robot into the room and shut off the gas valve.

HOW TO USE THIS GUIDE

This 4-page overview is for facilitators only. It contains the mission structure, team minimum standards, facilitation levers, crisis scripts, and debrief prompts. Each of the 7 teams receives their own Mission Brief separately — those are team-facing. A full mission timeline is also available at https://ogvbuildmo.tiiny.site/

STRUCTURE AT A GLANCE

| Hour | Day 1 — Design & Build | Day 2 — Integrate & Test | Day 3 — Mission Day |
| --- | --- | --- | --- |
| HR 1 | Glenn intro + scenario brief | Morning debrief — review Day 1 | Mission briefing + Go/No-Go |
| HR 2 | Team self-organisation | Close gaps; Chief Engineer CAD session | Pre-flight checks all teams |
| HR 3 | Preliminary Design Review (PDR) | Integration: Tank + Arm mounted on AU build | Dress Rehearsal on Kelsey's robot |
| HR 4 | ☀️ LUNCH | ☀️ LUNCH | ☀️ LUNCH + Kelsey positions robot |
| HR 5 | Begin build — all teams | Full system test on AU build | 🚨 MISSION GO |
| HR 6 | Build + first Comms relay to Kelsey | Fix issues, iterate | 🚨 ARM DEPLOYMENT + VALVE |
| HR 7 | EOD check + progress doc | Go/No-Go + first pilot of Kelsey's robot | Mission Debrief |
| HR 8 | OUTBRIEF | OUTBRIEF | 🎉 FINAL OUTBRIEF + Celebration |

TEAM MINIMUM STANDARDS — DAILY HARD FLOORS

If any team misses their floor, escalate immediately. These are minimums — not targets.

| Team | End of Day 1 | End of Day 2 | Day 3 Mission Ready |
| --- | --- | --- | --- |
| INTEGRATION | Dependency map live. AU ahead of Kelsey. | Gap list assigned. Go/No-Go data compiled. | Mission Director confirmed. Callout protocol rehearsed. |
| TANK | Chassis built. Pi mounted. H-bridge wired. | Remote drive of Kelsey's tank confirmed via RPC. | Dress rehearsal passed. Workarounds ready. |
| ARM | All servos wired to Arduino Mega. Servos 1–3 tested. Servo count confirmed. | All DOF remote-controlled via Pilot GUI. | Preset positions locked. Dress rehearsal passed. |
| GRIP | First print queued or running. Valve fit concept discussed. | Gripper fitted to arm. Valve engagement tested. | Valve turned in dress rehearsal. Backup staged. |
| POWER | Voltage map exists. No pin-powered servos. | Full load test passed. Kelsey's batteries charged. | All systems verified stable before mission start. |
| COMMS | RPC link live by HR6. First relay sent to Kelsey. | Kelsey's build matches AU build on camera. | Dress rehearsal relay complete. Escalation script rehearsed. |
| PILOT | GUI framework up. Tank drive controls functional. | First full remote pilot of Kelsey's robot done. | Full dress rehearsal complete. Latency tuned. |

FACILITATION LEVERS — USE PROACTIVELY, NOT REACTIVELY

Scale difficulty in real time. Check team progress every 30 min during build phases.

Day 1

| 🟢 MAKE IT EASIER Pre-flash MicroSD cards with Pi OS + RPC config Provide pre-wired breadboard power bus for Power team Issue servo wiring diagram to Arm team Shorten PDR to 4 min per team to protect build time Pre-print first gripper iteration to save Grip time | 🔴 MAKE IT HARDER Introduce component shortage — one part missing per team; must negotiate Require all Kelsey instructions to be written, not spoken Add timed relay rule: Comms must relay each step in under 60 seconds or resend Glenn rejects one PDR design; team must revise and re-present |
| --- | --- |

Day 2

| 🟢 MAKE IT EASIER Deploy pre-built tank/arm/GUI backups at HR3 deadline — do not wait for teams to ask Assign facilitator to quality-check Comms relay in real time Provide pre-printed gripper if 3D print fails Reduce arm to minimum DOF if 6DOF is blocked | 🔴 MAKE IT HARDER Introduce 5-minute RPC signal drop — teams must recover independently Power team discovers a "faulty fuse" — diagnose and replace under time pressure Kelsey reports a build discrepancy — Comms must resolve without showing AU robot Add mission constraint: robot must complete route in under 8 minutes |
| --- | --- |

Day 3

| 🟢 MAKE IT EASIER Simplify Room 2 path — remove one obstacle Allow Kelsey to manually steady the robot during arm deployment Expand mission time window by 15 minutes Reduce valve turn requirement (90° instead of full close) | 🔴 MAKE IT HARDER Degrade RPC to video-only for 3 minutes — Pilot must adapt without audio Comms loses audio for 60 seconds — Pilot must hold position Introduce secondary obstacle not present in dress rehearsal |
| --- | --- |

GO / NO-GO DECISION — DAY 2 HR7

| Question | GO Criteria | NO-GO Action |
| --- | --- | --- |
| Remote-drive Kelsey's tank? | Confirmed — Pilot has driven it | Overnight troubleshoot or simplify chassis |
| Remote-control Kelsey's arm? | All DOF responding via GUI | Reduce to essential DOF only |
| Gripper engages valve? | Tested on real device | Adjust design or use manual backup |
| Power lasts full mission? | Endurance confirmed | Plan shorter mission window |
| Comms reliable end-to-end? | Backup channel also ready | Set up backup channel tonight |
| GUI works on Kelsey's robot? | Tested directly on her system | Overnight fix or swap to backup GUI |

CRISIS SCRIPTS — ESCALATION SEQUENCE

Let teams work crises for 90 seconds before intervening. Exception: servo power wiring (intervene immediately).

🚨 RPC Link Fails

1. Comms: attempt reconnect via primary channel (60 sec) 2. Comms: switch to pre-configured backup channel 3. If backup fails: use any available channel (Teams, phone, email) + maintain GUI control 4. Facilitator intervenes only after 3 min unresolved: provide backup hotspot

🚨 Servo Power Wiring (INTERVENE IMMEDIATELY)

1. Stop the team. Point to the power diagram. 2. Arm + Power: rewire to dedicated 5V 5A+ BEC — never Arduino/Pi pins 3. Verify common ground shared with buck converter before power-on 4. Log near-miss — use in debrief

🚨 3D Print Fails or Wrong Dimensions

1. Grip: re-check CAD against mock valve measurements (1 person, 10 min) 2. If wrong: correct and queue re-print immediately — no waiting for consensus 3. Facilitator: deploy backup pre-printed gripper if no time 4. Chief Engineer documents correction and relays to Kelsey via Comms

🚨 Battery Brownout During Mission

1. Power: call battery level immediately via Comms 2. Integration: can mission complete within remaining power? 3. If YES: Pilot accelerates to valve, skips non-critical manoeuvres 4. If NO: abort run; Kelsey swaps to staged backup battery; add 10 min to clock

🚨 Motor or Servo Failure Mid-Mission

1. Pilot: hold position, report anomaly via Comms 2. Tank/Arm: diagnose from AU build — wiring or mechanical? 3. Integration: decide within 2 minutes — continue with reduced capability or abort 4. Facilitator: allow 5-min pause for remote diagnostic if unresolvable

DEBRIEF STRUCTURE — ALL OUTBRIEFS + DAY 3 HR7

| Phase | Duration | Facilitator Prompt |
| --- | --- | --- |
| 1. Facts | 10 min | "What happened? Walk me through the timeline." — Events only, no opinions yet. |
| 2. Feelings | 5 min | "What was the moment of highest stress? What surprised you?" |
| 3. Findings | 15 min | "What worked, specifically? What broke down and why?" — Draw out systemic causes. |
| 4. Futures | 10 min | "If you ran this again tomorrow, what's the one thing you'd change?" |
| 5. Transfer | 10 min | "Where does this show up in your real work? What does Kelsey's experience teach us?" |

TEAM DEBRIEF QUESTIONS — DAY 3 HR7

| Team | Core Debrief Question |
| --- | --- |
| INTEGRATION | When did you first know something was off-track? What was your decision-making process under time pressure? |
| TANK | How did remote control feel vs. your AU build? What would you build differently for a real mission? |
| ARM | Where did the arm behave differently on Kelsey's robot vs. yours? What's the lesson about specification vs. execution? |
| GRIP | Did the valve device work as designed? What assumption about the real environment turned out to be wrong? |
| POWER | Did the power budget hold? What would you change about your safety margins? |
| COMMS | What was the hardest thing to relay? How did Kelsey's experience change based on your instruction quality? |
| PILOT | How did the GUI perform under pressure? What does flying someone else's robot from thousands of miles away actually feel like? |

DAILY OUTBRIEF PROMPTS

| Every Outbrief → "Where are you vs. where Kelsey is — what's the gap?" → "What's the one thing that, if it fails tomorrow, kills the mission?" → "What’s your team need from another team first thing tomorrow?" | Day 2 Outbrief Only → "We are GO / NO-GO because..." — be direct and specific → "Kelsey: here’s your exact position for tomorrow morning." → "If X fails during the mission tomorrow, here is what we do." |
| --- | --- |

💡 FACILITATOR PRINCIPLE: The mission is the vehicle, not the destination. Teams learning to communicate across distance, translate expertise into actionable instructions, and operate under ambiguity IS the learning. When in doubt: let them struggle a little longer than feels comfortable. The debrief is where the gold is.

Operation Gas Valve  ·  Facilitator Guide  ·  Internal Use Only  ·  Not for distribution to participants

OPERATION GAS VALVE MISSION BRIEF — INTEGRATION TEAM Cross-team coordination, dependency tracking, Go/No-Go A full mission timeline overview is available at OGVbuildmo.tiiny.site

ℹ️ You are the nervous system of this mission. Your job is not to build but to make sure everyone else can build and that AU is always ahead of Kelsey. A full mission timeline is available at https://ogvbuildmo.tiiny.site/

## Mission-Critical Role

Integration owns the dependency map, the daily status, and the Go/No-Go call at Day 2 HR7. If two teams are blocking each other and don't know it — its integrations fault.

## Common Failure Mode

Getting siloed. Integration’s value is full-system visibility. If you’re focused on a single task, you will miss the full picture. Circulate, identify problems, and enable other teams - your role is coordination, not construction.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| End of Day 1 | Dependency map exists and is visible. Build-then-relay workflow defined. AU is confirmed 2+ steps ahead of Kelsey at end of HR6. |
| End of Day 2 | Gap list visible and teams are progressing. Go/No-Go data compiled and ready for HR7 decision. |
| Day 3 Mission Ready | Mission Director role confirmed. All teams on station. Cross-team callout protocol rehearsed. Milestone tracking live. |

## Verification Checks

| ☐ | Can you show the facilitator the dependency map right now? |
| --- | --- |
| ☐ | Is AU at least 2 steps ahead of Kelsey at all times? |
| ☐ | Is the gap list visible and being actively worked (Day 2)? |
| ☐ | Can Integration call out mission milestones in real time (Day 3)? |

## Day 1 Focus

HR 2 — Team Formation

Watch which teams are struggling to self-organize. Flag cross-team dependencies early.

HR 3 — PDR

Track all risks on a visible board. Ensure each team articulates their top 2 risks. Capture cross-team dependencies ASAP.

HR 5–7 — Build Phase

Circulate. Confirm AU build stays 2+ steps ahead of Kelsey. Check relay quality - are Comms instructions clear?

## Day 2 Focus

Own the Go/No-Go data. By HR6+30, start collecting answers to every Go/No-Go question — don't wait until HR7.

| Go/No-Go Question | GO Criteria |
| --- | --- |
| Remote-drive Kelsey's tank? | Confirmed — Pilot has driven it |
| Remote-control Kelsey's arm? | All 6 DOF responding via GUI |
| Gripper engages valve? | Tested on real device |
| Power lasts full mission? | Endurance calculated + confirmed |
| Comms reliable end-to-end? | Backup channel also ready |
| GUI works on Kelsey's robot? | Tested directly on Kelsey's system |

## Day 3 Focus

You are Mission Director. Call out milestones in real time. Run the escalation if something breaks. Brief every team on their first 30 minutes before HR1 ends.

## Outbrief Prompts (Use These Each Day)

- Where are you vs. where Kelsey is — what's the gap?

- What's the one thing that, if it fails tomorrow, kills the mission?

- What does your team need from another team first thing tomorrow?

## Debrief Question (Day 3)

ℹ️ When did you first know something was off-track? What was your decision-making process under time pressure?

OPERATION GAS VALVE MISSION BRIEF — TANK TEAM Chassis build, motor driver wiring, Pi-to-H-bridge PWM control, drive integration A full mission timeline overview is available at OGVbuildmo.tiiny.site

The tank is the mobility platform and mounting point . This team must work closely with every other team to ensure all systems operate seemlesly.

## Common Failure Mode

Motor calibration is a software fix. Check H-bridge IN1/IN2/ENA pin logic first. Adjust PWM duty cycle values and speed ramp curves in the Pi control script before assuming a hardware fault. Most “calibration” issues are incorrect pin mappings or inverted polarity in code.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| Day 2 HR3 (Hard Deadline) | Tank driving remotely via Pi. If not met: swap to pre-built chassis immediately — do not keep debugging. |
| End of Day 2 | Full remote drive of AU tank confirmed. Remote drive of Kelsey's tank confirmed via Pi Connect by HR7. |
| Day 3 Mission Ready | Dress rehearsal passed. Motor anomaly callout rehearsed. Remote workarounds ready. |

## Verification Checks

| ☐ | Can Pilot drive the tank via GUI remotely? (Day 2) |
| --- | --- |
| ☐ | Can Pilot drive Kelsey's tank via GUI from the AU station? (Day 2 HR7) |
| ☐ | Tank navigated dress rehearsal course without issue (Day 3) |

## Hard Deadline: Day 2 HR3

⚠️ If the tank is not driving remotely by HR3, the facilitator will deploy a pre-built chassis. transition to focusing on Kelsey's build and the delay.

## Day 1 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Inventory BOM: chassis, motors, L298N H-bridge, Pi 4, USB cable, power leads. Flag shortfalls to Integration. |
| 2 | Plan out mounting locations for all system components |
| 3 | Wire H-Bridge: OUT1/OUT2 → left motor, OUT3/OUT4 → right motor. Identify if the polarity is incorrect (Motors spin in wrong direction), then correct it. |
| 4 | Upload test script to Pi. Verify forward, reverse, left-spin, right-spin via direct PWM commands. |
| 5 | Document PWM pin assignments and calibration values. Pass wiring diagram to Comms for Kelsey relay. |

## Day 2 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Full Pi Connect remote drive test for AU build: GUI → Pi → PWM → H-bridge → motors. Verify latency and directional response match expectations. |
| 2 | Coordinate with ARM team: confirm arm base mounting pattern aligns to chassis top plate.. |
| 3 | Relay tank build steps to Comms for Kelsey's build. |
| 4 | By HR7: Pilot drives Kelsey’s tank via Pi Connect. Confirm H-bridge responds identically to AU — check for PWM calibrationbetween builds. |

## Backup Interventions (Facilitator May Deploy)

| If... | Backup |
| --- | --- |
| Tank not driving | Debug motor/H-bridge wiring first, then swap pre-built chassis |
| Motor brownout | Check H-bridge power connections; verify separate supply |
| Pi not connecting | Re-flash SD card from backup |

## Day 3 Mission Sequence

- Pre-flight: Remote drive confirmed on Kelsey's tank via RPC

- Dress rehearsal: Navigate full course, note any differences from AU build

- HR5 Mission: Pilot navigates Room 2 — you are on standby for motor anomaly callouts

- Callout if motor behaves unexpectedly — do not stay silent

## Debrief Question (Day 3)

ℹ️ How did remote control feel vs. your AU build? What would you build differently for a real mission?

OPERATION GAS VALVE MISSION BRIEF — ARM TEAM 6DOF servo control via Arduino Mega, PWM signal routing, end-effector integration A full mission timeline overview is available at OGVbuildmo.tiiny.site

The arm is driven by servos via Arduino Mega PWM outputs. Each servo requires a dedicated PWM signaland power is delivered from a 5V 20A rail via the buck converter.

## Common Failure Mode

The servos can only rotate between 0 and 180 degrees. If improperly installed, the arm will lack full range of motion. Additionally, if the software does not include limits, the arm will move in such a way that will damage the rest of the system. Finally, the full arm requires a dedicated power solution. The Arduino can power a single motor for testing, but the full arm with multiple servos requires a dedicated power solution, else the Arduino will be damaged.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| End of Day 1 | All servos identified and wired to Arduino Mega. Arm tested for range of motion by individually testing each servo . Team must confirm and document final servo count decision by end of Day 1. |
| Day 2 HR3 (Hard Deadline) | The arm moves through code control. If not: swap to pre-wired arm or reduce to minimum viable servos. |
| End of Day 2 | Arm mounted on tank. All servos responding to code. Remote arm control via GUI confirmed by HR7. |
| Day 3 Mission Ready | Preset positions locked for valve engagement. Dress rehearsal passed. Arm reached valve position. |

## Verification Checks

| ☐ | Do servos respond to Arduino commands? (Day 1) |
| --- | --- |
| ☐ | All servos moving under code? (Day 2 HR3) |
| ☐ | Can Pilot move Kelsey's arm via GUI from AU station? (Day 2 HR7) |
| ☐ | Arm reached valve position in dress rehearsal (Day 3) |

| Rule | Detail |
| --- | --- |

## Day 1 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Label servos per joint. Map each to a dedicated Arduino Mega PWM pin (D2–D13) and connect. |
| 2 | . Decide where to mount the arm |
| 3 |  |
| 4 | Establish the limits of each joint to ensure the arm will not damage the rest of the system |
| 5 | Relay arm build steps to Comms for Kelsey's build. |

## Day 2 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Complete PWM test and range-of-motion logging for servos. Write control code for all servos in Arduino sketch. |
| 2 | Bolt arm base to chassis top plate. Route servo cables to avoid pinch points across full travel range. |
| 3 | Map Pilot GUI axis controls to Arduino serial commands. Test end-to-end: GUI input → Pi serial → Arduino Mega → PWM → servo. |
| 4 | Chief Engineer CAD session (HR2): confirm end-effector bolt pattern, shaft diameter, and torque rating. Pass interface spec to Grip team before their CAD session. |
| 5 | By HR7: Pilot must move Kelsey's arm via GUI. Confirm directly. |

## Crisis: Servo Conflict or Power Spike

- Bring servos online one at a time. Monitor buck converter output voltage — sag below 4.8V indicates overcurrent on the 5V rail.

- If PSU voltage sags under combined load: increase PWM ramp time in code to reduce inrush, or sequence servo movements to limit simultaneous current draw.

## Day 3 Mission Sequence

- Pre-flight: All servos move via GUI

- Dress rehearsal: Arm reaches valve engagement position

- Preset positions: Lock in Home, Approach, Engage positions before mission

## Debrief Question (Day 3)

ℹ️ Where did the arm behave differently on Kelsey's robot vs. yours? What's the lesson about specification vs. execution?

OPERATION GAS VALVE MISSION BRIEF — GRIP TEAM 3D-printed end-effector design, CAD tolerancing, valve-engagement mechanism A full mission timeline overview is available at OGVbuildmo.tiiny.site

You are designing an end-effector to interface with a specific valve handle geometry, attach to the arm’s end-effector mount, and transmit sufficient torque to actuate the valve. Print in PLA/PETG, measure the mock valve to ±0.5mm before modelling, and test-fit before final print.

## Common Failure Mode

Dimensional mismatch between the printed end-effector and the real valve handle geometry. Always verify CAD model against physical mock valve measurements (±0.5mm tolerance) before committing to a full print. A wrong-dimension print wastes 45–90 min of mission-critical build time.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| End of Day 1 | At least one gripper concept sketched and started. First print queued or underway. Valve adapter concept discussed with Arm team. |
| Day 2 HR3 (Chief Engineer CAD) | Gripper designs in CAD. Complete Chief Engineer session. If no time: use backup gripper design from facilitator. |
| End of Day 2 | Printed gripper fitted to arm tip on AU build. Valve engagement tested on mock valve. Kelsey's gripper assembled. |
| Day 3 Mission Ready | Valve turned in dress rehearsal. Backup device staged. Engagement sequence confirmed with Pilot team. |

## Verification Checks

| ☐ | Is the 3D printer running or queued? (Day 1) |
| --- | --- |
| ☐ | Does the gripper fit the arm and engage the mock valve? (Day 2) |
| ☐ | Did the gripper engage the valve remotely in dress rehearsal? (Day 3) |
| ☐ | Is a backup manual device staged and ready? (Day 3) |

## Chief Engineer Role (Day 2 HR2)

One Grip team member is assigned Chief Engineer by the facilitator. This person leads the CAD session and ensures:

- Gripper dimensions match the mock valve exactly

- Attachment point matches Arm team's end-effector specs

- Design can be relayed to Kelsey for her build

- Corrected dimensions are logged and relayed via Comms

## Day 1 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Measure mock valve handle: engagement diameter, depth, handle width, and estimated actuation torque. Record all dims to nearest 0.5mm — these are your CAD inputs. |
| 2 | Draft gripper geometry in CAD or dimensioned sketch. Confirm bolt pattern/shaft interface with Arm team end-effector spec. Check jaw travel clearance needed for valve approach. |
| 3 | Commit to a CAD model and begin slicing. Use 0.2mm layer height, 30%+ infill for structural parts. Print, test, revise — do not iterate only in CAD. |
| 4 | Relay gripper concept to Comms so Kelsey understands what she'll be building. |

## Day 2 Build Sequence

| Step | Task |
| --- | --- |
| 1 | Chief Engineer CAD session (HR2): lock final gripper dims and arm interface spec. Start print immediately — print time is on the critical path. |
| 2 | During print: prep mounting hardware (M3 bolts, standoffs). Pre-assemble non-printed components. Confirm arm end-effector mount is ready to receive gripper. |
| 3 | Test fit on mock valve: jaw clearance on approach, engagement depth, and torque transmission. If geometry is off, correct CAD and reprint — do not force fit. |
| 4 | Bolt gripper to arm end-effector mount. Apply hand torque load — no flex or slip at joint. Verify servo cable routing clears through full jaw travel. |
| 5 | Relay gripper assembly steps to Comms for Kelsey's build. |

## Crisis: Print Fails or Wrong Size

- Re-check CAD dimensions against mock valve immediately (1 person, 10 min)

- If wrong: correct and queue re-print immediately — do not wait for team consensus

- If no time for reprint: facilitator deploys backup pre-printed gripper

- Chief Engineer documents the correction and relays to Kelsey's build via Comms

## Day 3 Mission Sequence

- Pre-flight: Gripper engages valve device remotely — confirm force OK

- Dress rehearsal: Full engagement sequence with Pilot — valve turned

- HR6 Mission: Guide Pilot to engagement position. Confirm grip. Activate valve device.

- Backup device staged and accessible throughout mission

## Debrief Question (Day 3)

ℹ️ Did the valve device work as designed? What assumption about the real environment turned out to be wrong?

OPERATION GAS VALVE MISSION BRIEF — POWER TEAM Power architecture, voltage regulation, H-bridge and servo rail management A full mission timeline overview is available at OGVbuildmo.tiiny.site

You own the power architecture for both builds. Three rails: 5V logic (Pi, USB-C), 7–12V motor (H-bridge, LiPo/NiMH), and 5V servo (buck converter output). Rails must be isolated. All GNDs must be tied to a common bus. Any floating-ground or shared-rail condition will cause brownouts, erratic behaviour, or permanent component damage. Document everything; verify with a multimeter.

## Common Failure Mode

Servo current through Arduino 5V or Pi GPIO pins. The Arduino 5V rail is rated ~500mA total; a single MG996R draws up to 2.5A under stall. Six servos will permanently damage the board or Pi. All servo power must route through the dedicated 5V 5A+ buck converter output. Physically inspect both AU and Kelsey’s build — do not assume.

## Daily Minimum Standards (help fill in please!)

| Day | What You Must Deliver |
| --- | --- |
| End of Day 1 |  |
| Day 2 HR3 |  |
| End of Day 2 |  |
| Day 3 Mission Ready |  |

## Verification Checks

| ☐ | Is the power map documented and visible? (Day 1) |
| --- | --- |
| ☐ | No servos on Pi or Arduino pins — verified by physical inspection? |
| ☐ | System runs 20 min continuous without brownout? (Day 2) |
| ☐ | Power systems stable and verified before mission start (Day 3) |

## Voltage Map — Complete This Day 1

| Subsystem | Required Supply |
| --- | --- |
| Raspberry Pi | 5V 3A via USB-C power bank — dedicated, not shared |
| Arduino Mega | 7–12V via barrel jack (recommended) or USB — verify adequate current rating |
| Drive motors (H-bridge) | Dedicated 7–12V LiPo/NiMH via L298N. Verify Vmotor does not exceed H-bridge absolute maximum. |
| Servos (ALL 6) | Dedicated 5V 5A+ BEC or PSU — NEVER from Pi/Arduino |
| Common ground | All grounds connected: servo supply GND ↔ buck converter GND |

## Crisis: Battery Brownout During Mission

- Call battery level immediately via Comms to Kelsey

- Integration decides: can mission complete within remaining power?

- If YES: Pilot accelerates to valve engagement, skips non-critical maneuvers

- If NO: abort current run; Kelsey swaps to staged backup battery; add 10 min to clock

## Debrief Question (Day 3)

ℹ️ Did the power budget hold? What would you change about your safety margins?

OPERATION GAS VALVE MISSION BRIEF — COMMS TEAM Raspberry Pi Connect (RPC) link, relay of build instructions to Kelsey A full mission timeline overview is available at OGVbuildmo.tiiny.site

ℹ️ You are the spine of this mission. Kelsey can only do what you tell her. If your instructions are unclear, she builds the wrong thing. If your link is down, the mission stops. Everything depends on you.

## Common Failure Mode

Jargon-heavy instructions Kelsey can't follow. She is not an engineer. Every instruction must be plain language, one step at a time, with confirmation before moving on. The facilitator will audit your relay quality every 30 minutes.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| Day 1 HR6 (Hard Deadline) | RPC link MUST be live. First relay of Tank build steps sent to Kelsey. Jargon-to-plain-language protocol established. |
| End of Day 1 | RPC confirmed live and stable. Kelsey has received and executed first build phase instructions. |
| End of Day 2 | Kelsey's build matches AU build visually on camera. All relay steps confirmed received and executed. Ready for Day 3 live ops. |
| Day 3 Mission Ready | Dress rehearsal relay complete without a gap. Crisis escalation script rehearsed. Kelsey understands full mission plan. |

## Verification Checks

| ☐ | Can you see and hear Kelsey via RPC right now? (Day 1 HR6) |
| --- | --- |
| ☐ | Is Kelsey's robot visually matching the AU robot on camera? (Day 2) |
| ☐ | Comms completed dress rehearsal relay without a gap? (Day 3) |

## Comms Chain — Follow This Every Time

Technical team → Comms (translate to plain language) → Kelsey confirms "done" or asks clarifications → next step

ℹ️ Kelsey should never be improvising. She executes exactly what she's told, nothing more.

## RPC Setup Checklist (Day 1 HR6 Hard Deadline)

| ☐ | Primary RPC channel established and stable |
| --- | --- |
| ☐ | Audio clear — no lag >500ms |
| ☐ | Video quality sufficient to see Kelsey's build clearly |
| ☐ | Backup RPC channel configured and tested |
| ☐ | Backup communication channel confirmed with Kelsey |

## Crisis: RPC Link Fails

- Attempt reconnect via primary channel — give it 60 seconds

- Switch to pre-configured backup Pi channel

- If backup fails: use any available channel (Teams, phone, email); maintain GUI control

- Do NOT call for facilitator help in first 90 seconds — work it yourselves

- If unresolved after 3 minutes: flag for facilitator

## Crisis: Kelsey Can't Follow Instructions

- Pause. Is it jargon? A missing step? Wrong order?

- Translate one step at a time; confirm each before proceeding

- Allow Kelsey up to 3 clarifying questions for any critical build step

- Log all failures — they will be used in debrief

## Day 3 Mission Role

- Live relay confirmed before mission start

- Announce MISSION GO at HR5

- Call MISSION COMPLETE when gas valve is shut off

- Manage any communication gaps during mission — do not stay silent

## Debrief Question (Day 3)

ℹ️ What was the hardest thing to relay? How did Kelsey's experience change based on your instruction quality?

OPERATION GAS VALVE MISSION BRIEF — PILOT TEAM GUI development, RPC control loop, latency compensation, mission execution A full mission timeline overview is available at OGVbuildmo.tiiny.site

You are the remote operator. Control signals originate in your Graphical User Interface (GUI), travel from your Pi to Kelsey’s Pi via Pi Connect, and communicates through USB to the Arduino Mega. Every input has latency. Design your GUI sensitivity around the measured round-trip delay — build for zero-lag target, then add a buffer margin for the long-haul Australia connection.

## Common Failure Mode

Building the GUI against the AU robot only. Kelsey’s build may have different serial port assignments, minor wiring offsets, or PWM calibration differences. Every GUI control must be tested directly on Kelsey’s robot by Day 2 HR7. If mappings differ, update GUI to match her build — not AU.

## Daily Minimum Standards

| Day | What You Must Deliver |
| --- | --- |
| End of Day 1 | GUI framework set up (not complete). Tank drive controls functional in GUI. Control layout defined. Simulator access confirmed. |
| Day 2 HR3 (Hard Deadline) | Basic tank controls functional in GUI. If not: swap to pre-built GUI immediately. |
| End of Day 2 | GUI controls tank + arm + grip on Kelsey's robot. First full remote pilot run completed. Latency tuning started. |
| Day 3 Mission Ready | Full dress rehearsal sequence completed. GUI latency tuned. Mission controls confirmed. Gas valve test completed with Grip team. |

## Verification Checks

| ☐ | Does clicking the GUI move anything on the AU tank? (Day 1) |
| --- | --- |
| ☐ | Can Pilot drive Kelsey's tank via GUI from AU station? (Day 2 HR7) |
| ☐ | Can Pilot move Kelsey's arm via GUI from AU station? (Day 2 HR7) |
| ☐ | Pilot navigated, deployed arm, and turned valve in dress rehearsal (Day 3) |

## GUI Requirements — Build in This Order

| Priority | Control |
| --- | --- |
| 1 — Day 1 | Tank drive: WASD or joystick → Pi GPIO PWM → L298N H-bridge → DC motors |
| 2 — Day 2 | Arm: 6-axis sliders or joystick → Pi serial → Arduino Mega PWM → MG996R servos |
| 3 — Day 2 | Gripper: open/close toggle → serial command → servo 6 PWM pulse |
| 5 — Day 2 | E-STOP: single key/button → immediately zeros all PWM outputs |
| 6 — Day 2 | Latency / signal quality indicator |

## Simulator Training (Day 1)

All Pilot team members must complete simulator training during Day 1 build hours. Prioritize:

- Low-speed differential drive in a confined corridor — practice tight turns at varying PWM rates

- Arm deployment from stationary chassis — learn servo response curves before adding chassis movement

- End-effector precision targeting — valve handle has a small engagement window; practice consistent approach heading

- Operating with high-latency or partial camera feed — build spatial awareness without full visual fidelity

## First Pilot Run on Kelsey's Robot (Day 2 HR7)

This is the most important test of Day 2. Complete this sequence:

| Step | Confirm |
| --- | --- |
| Tank: drive forward 1 metre | Correct PWM response via RPC — check for latency delta vs AU build |
| Tank: turn left, turn right | No direction reversal or lag spike — verify H-bridge pin mapping matches AU |
| Arm: cycle all DOF | All joints respond via Arduino serial — no servo conflict or PSU voltage sag |
| Gripper: open and close | Full jaw travel, no binding — confirm PWM min/max match physical stops |
| Emergency stop | All PWM outputs zero within one control cycle — confirm E-STOP works over RPC |

## Latency Management

- Measure round-trip control latency (GUI input → Pi → Arduino → servo response). Document the value. Tune GUI input sensitivity to compensate — build in a margin buffer above the measured baseline.

- Report measured latency to Integration before Go/No-Go. They need it to assess mission viability and set pilot expectations.

## Day 3 Mission Sequence

- Pre-flight: All GUI controls confirmed on Kelsey's robot

- Dress rehearsal: Full sequence — navigate, deploy arm, turn valve — timed

- HR5 Mission: Navigate Room 2 to valve position

- HR6 Mission: Arm deployment → engagement → TURN THE VALVE

- Call Comms when valve engagement is confirmed

## Crisis: GUI Breaks During Mission

- Call GUI failure immediately via Comms — do not attempt silent recovery mid-mission.

- Switch to backup GUI. If it uses different control mappings, brief Integration before resuming.

- Last resort: direct serial terminal to Arduino Mega for arm; direct Pi GPIO script for drive.

- Integration assesses residual control capability and calls continue or abort.

## Debrief Question (Day 3)

ℹ️ How did the GUI perform under pressure? What does flying someone else's robot from thousands of miles away feel like?